Standard Analytical Method

Determination of Soil Carbon Content

Elemental Analysis by combustion method

Contents

1	Introduction	3

2	Scope	3

3	Principle	3

4	References	3

5	Apparatus and laboratory equipment	3

6	Reagents	3

7	Standards and reference materials	4

7.1	Standards – certified materials	4

7.2	Internal standards	4

7.3	Analytical reference materials	4

7.4	Use of standards and reference materials	4

8	Health & Safety	5

9	Soil sample preparation	5

10	Elemental Analysis Procedure	6

10.1	Calibration and performance checks of the Elemental Analyser apparatus	6

10.2	Determination of carbon content in soils	6

10.2.1	Total carbon content	6

10.2.2	Soil organic carbon content	6

11	Calculations	6

12	Quality Management	7

12.1	Precision test	7

12.2	Trueness test	7

12.2.1	Recovery	7

12.2.2	Limits of Detection	7

12.2.3	Interlaboratory comparison	8

12.3	Quality Control chart	8

13	Revision History:	8

14	Document Approval:	8

# Introduction

The measurement of soil organic carbon is necessary for the quantification of soil carbon stocks.

Elemental analysis by combustion is considered an accurate and reproducible method for the measurement of soil organic carbon when carbonates are removed prior to analysis.

# Scope

This standard operating procedure (SOP) describes the procedure for the analysis and quantification of soil organic carbon (SOC) in soil samples with the removal of carbonates using an acid solution prior to dry combustion using an elemental analyser.

# Principle

Elemental analysis using dry combustion measures total carbon in soils by detecting the amount of  carbon dioxide (CO2) released from soils which undergo combustion at temperatures 900oC or higher in an atmosphere free from CO2. The amount of CO2 released is measured using solid state infrared absorption.

Elemental analysis requires small sample weights (typically less than 150 mg) and therefore soil samples need to be well homogenized and adequately sub-sampled to obtain representative measurements of soil organic carbon.

The addition of acid to the sub-sample prior to combustion will remove carbonates present in the soil. The efficiency of carbonate removal reflects the type and concentration of acid and the time allowed for the acid to work.

# References

- ISO/IEC 17025 Testing and calibration laboratories.

- BS 7755: Section 3.8: 1995 ISO 10694:1995 Soil quality – chemical methods – determination of organic and total carbon after dry combustion (elementary analysis).

# Apparatus and laboratory equipment

- Elemental combustion analyser

- Balances to weigh soil samples c. 150 mg +/- 0.01 mg

- Balance to weigh soil samples up to 3 kg to +/- 0.01 g

- Crucibles for sample combustion in elemental analyser

- Calibration weights for balances

- Standard laboratory glassware and other laboratory equipment

- Barcodes

- Barcode readers

- Pipettes

- Soil sieves - stainless steel 2mm

- Soil grinding equipment

- Ovens for sample drying at 70 oC to 120 oC

# Reagents

- Water

- Carrier gases of high purity (>99.99%) for the elemental analyser including O2.

- Absorbing agent required by the elemental analyser – as per manufacturers requirements

- Calibration reagents for elemental analyser - calcium carbonate (CaCO3) from two difference sources

- Hydrochloric acid

- Suspending agent, when required

Records for the purchase, preparation and use of reagents are maintained in Agricarbon Quality Management System.

# Standards and reference materials

Reference materials and internal standards are stored in the desiccator, where necessary, to avoid absorption of moisture, where necessary e.g. Calcium carbonate, CRM, internal standard soils when in use.

The use of standards and reference materials are detailed on the relevant SOPs.

## Standards – certified materials

The following materials are used as certified standards for quality management.

- Calcium carbonate from two difference sources

- Certified Reference Materials, which will range from 0.46 to 15% soil organic carbon.

Documentation associated with these reference materials are maintained in the Agricarbon Quality Management System and include analytical data, certificate of analysis and material safety data sheet, where relevant.

## Internal standards

The following materials are used as internal standards for quality management.

- Agricarbon Internal Reference Material Standard Soil against control charts.

- Agricarbon standard bulk soil:  air-dried, sieved to 20 mm and stored in air-tight containers

Documentation associated with the production of these internal standards are maintained in the Agricarbon Quality Management System and include collection, preparation, storage, analytical data and handling procedures.

## Analytical reference materials

The following are also included in analytical batches for elemental analysis as standards for quality management.

  - Blank crucibles

  - Acidified blanks crucibles

## Use of standards and reference materials

Routine use of all standards, reference materials and responsible staff are outlined in the Operator and Quality Checklists. Associated SOPs provide details on use.

Active documents are maintained in the Agricarbon Quality Management System .

Results from the use of the standards and reference materials are collated in the Agricarbon Quality Management System .

Analysis, interpretation and reporting of quality control data is the daily responsibility of the Agricarbon quality manager. The process for maintaining quality control is documented in the Agricarbon Quality Management System.

# Health & Safety

Agricarbon Health and Safety procedures detail the requirements for safety management in using this SOP, with relevant documentation available in the Agricarbon Quality Management System.

In summary:

- Appropriate safety precautions are necessary.

- All operators must be trained with appropriate PPE provided (e.g. gloves, lab coats and eye protection).

- Personal protection equipment must be worn when handling reagents, reference materials and soil samples.

- All documentation regarding hazardous substances is contained in the Agricarbon COSHH register.

- Consult the material safety data sheet (MSDS) for the relevant information on handling and use of reagents and reference materials and conduct additional risk assessments where necessary.

- Take necessary precautions when handling compressed gasses and high-temperature equipment. Follow the manufacturer’s safety guidelines when operating the elemental analyser.

- Samples, waste materials and  residues maybe toxic and must be disposed of properly.

# Soil sample preparation

There are two procedure routes for sample preparation.

Soil samples from the same project will always be prepared using only one of these approaches unless there is a documented reason for the analysis of project samples using both approaches.

Procedure A is a universal standard for the preparation of soil samples for elemental analysis.

- Soil samples are dried at a minimum of 75oC (up to a maximum of 120oC) for at least 2.5 hrs

- Samples are sieved to 2 mm and then ground to a fine powder.

- Samples are stored in air-tight containers prior to sub-sampling.

- Two representative sub-samples are taken from the prepared soil and weighed into individual sample crucibles to a target weight range

- Hydrochloric acid (10% v/v) is added to samples in crucibles.

- Crucibles are dried in an oven (75 - 105°C) for a minimum of 1.5 hrs prior to combustion in the elemental analyser.

SOP document Procedure A for sample preparation is available in Agricarbon Quality Management System.

- Two representative sub-samples are taken from the prepared soil and weighed into individual sample crucibles to a target weight range.

- Hydrochloric acid (10% v/v) is added to a representative sub-sample in a crucible.

  - 1 crucible will receive acid prior to drying and combustion

  - 1 crucible will not receive acid. This crucible is capped after oven drying prior to storage.

- Crucibles are dried in an oven (75 - 105°C) for a minimum of 1.5 hrs

  - 1 crucible with acid proceeds to combustion in the elemental analyser

  - 1 crucible without acid is capped and sent to storage.

SOP document Procedure B for sample preparation details this procedure and is available in Agricarbon Quality Management System.

# Elemental Analysis Procedure

## Calibration and performance checks of the Elemental Analyser apparatus

Calibration and performance checks of apparatus are carried out at the start of each shift with results recorded.

The analyser is calibrated as described in the manufactures manual using one of the substances listed in section 7.  Generally, this uses triplicate sub-samples of one CaCO3 sample followed by a performance check using the other CaCO3 sample.

If more than 1 of the performance checks fails then the calibration process is repeated.

Triplicate blank crucibles are run to determine the baseline.

Regular checks of performance are carried out using CRM, control samples and blanks which are incorporated in each batch of soil samples. The exact number and frequency of control and check samples depends on the method used and the performance of the elemental analyzer.

The daily routine for performance checks and responsible staff are outlined in the Operator and Quality Checklists.

SOP document for calibration and performance details the procedure and is available in Agricarbon Quality Management System.

## Determination of carbon content in soils

### Total carbon content

- Soil samples are analysed according to the manufacturer’s guidelines for elemental analysis of soils for total carbon.

- Manufacturers guidelines for elemental analysers in use are maintained in the Agricarbon Quality Management System.

### Soil organic carbon content

- Soil samples are analysed as for total carbon with the additional step of the addition of HCl and drying prior to elemental analysis, see Section 9.

- During organic carbon analysis, acidified blank crucibles are included to ensure not additional carbon is introduced through the acid treatment.

# Calculations

Total carbon and organic carbon are reported using the International Units System as: grams of C (g) per kilogram (kg) of soil, g/kg. Soil organic carbon is also reported as %, as a common unit required by clients.

Results are reported:

- on an oven-dry soil basis.

- to 2 decimal points (e.g. 0.01)

Procedure A

The elemental analyser presents the results as g/kg and therefore no additional calculations are required.

Procedure B

The results from the elemental analyser for organic carbon in the analysed sample (OC%) are used in a series of calculations to derive SOC%

The calculations are detailed in the relevant document in the Agricarbon Quality Management System.

# Quality Management

## Precision test

- The internal standard soil is run regularly throughout the analytical process with minimum of 1 internal standard soil per 25 soil  samples.

- Precision limits for all analysers are defined by analysing the collated data from the results of the internal standard soils over a number of batches for each analyser in use.

- The results from internal standard  soils are used to check precision  and precision limits on an on-going basis.

- All results are maintained in the Agricarbon quality management system.

- The Agricarbon quality manager is responsible for these on-going checks.

- If the precision test fails, the cause of the failure must be identified and corrective or preventive actions must be developed.
Actions required as a result of deviations from expected precision are detailed in the Operator and Quality Checklists.

## Trueness test

### Recovery

- At least two Certified Reference Materials with different levels of soil organic carbon are analysed as replicates on a regular basis.

- The results are used to check recovery  on an on-going basis. All results are maintained in the Agricarbon Quality Management System.

- The Agricarbon quality manager is responsible for these on-going checks.

- Recovery is considered acceptable if the results for the  is within the 95% confidence interval reported for the target value of the CRMs.

- If the recovery test fails, the cause of the failure must be identified and corrective or preventive actions

- must be developed.

- Actions required as a result of deviations from expected precision are detailed in the Operator and Quality Checklists

SOP for Recovery details this procedure and is available in Agricarbon Quality Management System .

### Limits of Detection

Limits of detection for each analyser are established by using a specific Certified Reference Material of low soil organic carbon content.

A series of CRM samples are run through the analysers to determine where the carbon can no longer be reliably determined.

The can be done by either through reducing the mass of CRM or by diluting the CRM by known amounts with an inert material. The process will be dependent on the analysers in use.

The Agricarbon Quality Manager is responsible for establishing the limits of detection.

All results are maintained in the Agricarbon Quality Management System .

SOP for Limits of Detection details this procedure which  is available in Agricarbon Quality Management System .

### Interlaboratory comparison

The laboratory participates in a interlaboratory proficiency test at least once a year.

Agricarbon Quality Manager coordinates this activity, reviews the results from the test and initiates actions where required.

All results are maintained in the Agricarbon Quality Management System.

If the results are questionable or unsatisfactory, the Agricarbon Quality Manager and relevant staff carry out an evaluation to implement corrective and preventive actions.

SOP for Interlaboratory Trials  details this procedure which  is available in Agricarbon Quality Management System .

## Quality Control chart

SOP for Quantity Control  details this procedure which is available in Agricarbon Quality Management System. In summary:

- The Agricarbon Quality Manager is responsible for establishing, maintaining and reviewing the quality control charts and in determining the requirements for action as a consequence of the results.

- Internal Standard Soils samples are used on a regular basis across sample batches to provide initial data to set limits in the quality control chart and then to provide  routine data for maintaining the quality control charts.

- Results for each analyser for the Internal Standard Soils are plotted continually in control charts.

- When results are out of the specified limits or if there is a consistent trend across results then a detailed an evaluation will be done.

- Causes for noncompliance will be determined wherever possible

- Corrective and preventive actions will be implemented.

# Revision History:

| Section / Step | Description of Change | Reason for Change | Reference |
| --- | --- | --- | --- |
| Version 1 | Final Edit | na |  |

# Document Approval:

| Role | Print Name | Signature | Date |
| --- | --- | --- | --- |
| Author: | Malcolm Stanger | Via DocuSign | 30/11/2022 |
| Reviewed By: | Helaina Black | Via DocuSign | 30/11/2022 |
| Approved By: | Stewart Arbuckle | Via DocuSign | 30/11/2022 |